# Water Quality

A clean water supply is vital to the health and wellbeing of the Urbenville, Woodenbong and Muli Muli Villages. Water undergoes regular and thorough testing at many points of the treatment and distribution system. Water samples are sent to accredited laboratories to ensure residents have a safe and secure drinking water supply.

Good, prolonged rain through to September meant that the weir continues to overflow from January to November 2021 and December 2022 to January 2023, March and April 2023 have seen a decline in rainfall. As a result, we have seen variations in water quality.

Figures 1 and 2: Water flowing in the Tooloom Creek and over the Tooloom Falls and Weir September 2022

Occasionally you may notice your water is a little different to usual. Read below for our recommended water quality solutions.

Discoloured or cloudy water

Discoloured or cloudy water is usually because of a change within your water pipes. An increase in water flow rate or swift change in direction of water flow in the pipes can stir up sediment. The sediment makes the water look discoloured but is harmless, and safe to drink.

What to do

To clear up the discolouration try running a garden tap closest to your water meter for around 2 minutes and then see if the water is clear when run into a glass. Remember to catch the running water in a bucket to use on your garden.

If the water doesn't clear, contact us via email Coucnil@tenterfield.nsw.gov.au

White water

When air is trapped inside pressurised water pipes, it is converted to tiny air bubbles which gives water a white or milky appearance. This water is still safe to drink.

Air can enter the water supply causing the discolouration during repairs to the pipe network.

What to do

Catch water in an open container and it will become clear within a few minutes. The bubbles will clear from the bottom of the glass upward

Discoloured water

Hard water is caused by a higher than usual concentration of calcium and magnesium salts in water. Water hardness levels are monitored on a regular basis in conjunction with Laboratory Services and reported as mg/L (milligrams per litre) of calcium carbonate.

Hard water

The water is safe to drink. The smell will disappear simply by leaving a jug of water uncovered in the fridge for a short period of time.

Taste or smell of chlorine

The water is safe to drink. If there is an odour or 'dirt' in the water, flush the tap for approximately 1 minute before using. This will flush the pipes out.

'Dirty' tasting and smelly water

# Urbenville Drinking Water Health Card

Health Cards will be prepared monthly and will report how our drinking water meets the quality levels set by the Australian Drinking Water Guidelines and NSW Health requirement in key areas.

We test for a range of water quality characteristics, guided by the Australian Drinking Water Guidelines. The guidelines:

- set the standards for good quality drinking water

- outline good practices for operating a water supply system

- help protect public health

- tell us how drinking water should look and taste.

The characteristics are categorised as physical, chemical and microbial.

| Physical | Chemical | Microbial |
| --- | --- | --- |
| Turbidity | Free chlorine | Pathogens |
| Total dissolved solids | Inorganic chemicals (dissolved salts) | Cyanobacteria |
| Conductivity | Organic compounds |  |
| pH |  |  |
| Hardness |  |  |
| Temperature |  |  |
| Dissolved oxygen |  |  |
| Colour |  |  |
| Taste |  |  |
| Odour |  |  |

Our Health Card will be reporting on the following items - based on Australian Drinking Water Guidelines (ADWG) and NSW Health

| Characteristics | Unit of measure | ADWG levels |
| --- | --- | --- |
| E. coli | MPN/100 mL | not detected in 100 mL |
| turbidity | NTU | 0.0 to 0.5 |
| true colour | HU | 15 |
| pH | pH units | between 6.5 - 8.5 |
| Total dissolved solids | Mg/L | 600mg/L |
| fluoride | mg/L | 1.5* |
| iron | mg/L | 0.3 |
| manganese | mg/L | 0.1 |

| Measures |  |
| --- | --- |
| 1 milligram (mg) | 0.001 gram (g) |
| 1 gram (g) | 1000 milligrams (mg) |
| 1 kilogram (kg) | 1000 grams (g) |
| 1 Litre (L) | 1000 millilitres (mL) |
| 1 millilitre (mL) | 0.001 Litres (L) |
| NTU | Nephelometric Turbidity Units |
| HU | Hazen Units (also referred to as TCU (True Colour Units) |
| MPN | Most Probable Number (MPN) is a method used to estimate the concentration of viable microorganisms in a sample |

Graph 1 Creek % Levels and Consumption

# Feedback

Residents are welcome to report any feedback or changes to their water appearance or quality by emailing council@tenterfield.nsw.gov.au

# Links:

Australian Drinking Water Guidelines

NSW Department of Industry supports local utilities (Councils) in providing water supply and sewerage services

Internal Documents:

The following documents can be found on Council’s Internet Page –

Water Supply (includes Water Restrictions)

Drought Management Plan

Water Conservation & Demand Management Plan